SMM1200 POWER SUPPLIES

150 W CHASSIS-MOUNTED

SWITCH MODE POWER SUPPLIES

DATA SHEET

SMM.1200 POWER SUPPLIES

150 W Chassis-Mounted Switch Mode Power Supply

TECHNICAL DETAILS

| INPUT | Voltage Range | 88 – 264 VAC |
| --- | --- | --- |
|  | Frequency Range | 47 – 63 Hz |
|  | Efficiency | 82% |
|  | Inrush Current | 35 A |
| OUTPUT | Voltage | 12 VDC Nominal |
|  | Current | 12.5 Amps |
|  | Rated Power | 150 Watts |
|  | Ripple | 180 mv p-p |
|  | Adjustment | 10.6 – 13.2 VDC |
| ENVIRONMENT | Temp Range | - 10 - + 60C |
|  | Humidity | 20-90% non-condensing |
| ANCILLIARY | Over-voltage protection; shut down o/p voltage, re-power on to recover. MTBF est. 286.7K hrs |  |
| STANDARDS | UL1012, En60950-1 |  |
| DIMENSIONS | 199 mm x 110 mm x 50 mm (L x W x H) |  |
| ENCLOSURE OPTIONS | 1) Chassis-Mounted |  |
|  | 2) Large (300 mm x 400 mm x 105 mm). |  |

| MODELS AVAILABLE | SMM1200L | 1 individual fused output |
| --- | --- | --- |
|  | SMM1210L | 10 individual fused outputs |
|  | SMM1220L | 20 individual fused outputs |

Power-Plex reserves the right to alter the above specification without notification to keep pace with technical innovation. January 2006.