SMM5000 POWER SUPPLIES

60 W CHASSIS-MOUNTED

SWITCH MODE POWER SUPPLIES

DATA SHEET

SMM.5000 POWER SUPPLIES

60 W Chassis-Mounted Switch Mode Power Supply

TECHNICAL DETAILS

| INPUT | Voltage Range | 85 – 264 VAC |
| --- | --- | --- |
|  | Frequency Range | 47 – 63 Hz |
|  | Efficiency | 76% |
|  | Inrush Current | 40 A / 230 VAC |
| OUTPUT | Voltage | 12 VDC Nominal |
|  | Current | 5 Amps |
|  | Rated Power | 60 Watts |
|  | Ripple | 120 mv p-p |
|  | Adjustment | 10.8 – 13.2 VDC |
| ENVIRONMENT | Temp Range | - 10 - + 60C |
|  | Humidity | 20-90% non-condensing |
| ANCILLIARY | Over-voltage protection; hiccup mode, auto-recovery after removal of fault. MTBF est. 316K hrs |  |
| STANDARDS | UL1012, UL1950 & EN60950 |  |
| DIMENSIONS | 159 mm x 97 mm x 38 mm (L x W x H) |  |
| ENCLOSURE OPTIONS | 1) Chassis-Mounted |  |
|  | 2) Compact (210 mm x 235 mm x 90 mm). |  |
|  | 3) Vertical (225 mm x 345 mm x 80 mm). |  |

| MODELS AVAILABLE | SMM5000C | 1 individual fused output |
| --- | --- | --- |
|  | SMM5005C | 5 individual fused outputs |
|  | SMM5010C | 10 individual fused outputs |
|  | SMM5015V | 15 individual fused outputs |
|  | SMM5020V | 20 individual fused outputs |

Power-Plex reserves the right to alter the above specification without notification to keep pace with technical innovation. January 2006.